ARCHITECTURAL OUTLINE SPECIFICATIONS

FOR DE LUCIA RESIDENCE

PREPARED BY

AR. REANNE DENISE PE

- MASONRY

  - Concrete Hollow Block (CHB)

    - Exterior Walls

      - 200 mm (8”) wide CHB with a minimum ultimate compressive strength (Fm’) of 450psi at 28 days;

      - Cells shall be filled solidly with cement mortar grout;

      - Plain cement plaster finish.

    - Exterior Walls

      - 150 mm (6”) wide CHB with a minimum ultimate compressive strength (Fm’) of 450psi at 28 days;

      - CHB shall be filled solidly with cement mortar grout;

      - Plain cement plaster finish.

    - Interior Walls

      - 100 mm (4”) wide CHB with a minimum ultimate compressive strength (Fm’) of 450psi at 28 days;

      - CHB shall be filled solidly with cement mortar grout;

      - Plain cement plaster finish.

- METALS

  - Stair Railing

    - 50 mm (2”) dia. X 2 mm X 6,000 mm TUBULAR STEEL

      - Finish: QDE Painted (color as prescribed by the Architect)

      - Location: All Stair Handrails

    - 38 mm (1 ½”) dia. X 1.5 mm X 6,000 mm TUBULAR STEEL

      - Finish: QDE Painted (color as prescribed by the Architect)

      - Location: All Stair Handrails Post

    - 15 mm (5/8”) dia. X 6,000 mm ROUND BAR

      - Finish: QDE Painted (color as prescribed by the Architect)

      - Location: All Stair Railing

  - Balcony Railing

    - 50 mm (2”) dia. X 2 mm X 2,000 mm TUBULAR STEEL

      - Finish: QDE Painted (color as prescribed by the Architect)

      - Location: All Balcony Handrails

    - 32 mm (1 1/4”) X 6 mm (1/4”) X 6,000 mm FLAT BAR

      - Finish: QDE Painted (color as prescribed by the Architect)

      - Location: Horizontal Flat Bars at All Balcony Railing

    - 38 mm (1 ½”)  X 38 mm (1 ½”) X 2,000 mm TUBULAR STEEL POST

      - Finish: QDE Painted (color as prescribed by the Architect)

      - Location: All Balcony Post

- WATER PROOFING

  - TOILET AND BATH

    - Flexible Cementitious Waterproofing, or approved equal

  - FAÇADE JOINTS

    - Acrylic Elastomeric Waterproofing, or approved equal

- ROOFING

  - MAIN ROOF SHEET

    - Rib Type Long Span Pre-painted G.I.  Sheets GA#24, or approved equal.

  - ROOFING ACCESSORIES

    - UGC Pre-painted Box Type Gutter, Flashing, Valley Gutter, Ridge Roll, all GA#24, or approved equal.

  - INSULATION

    - 25 mm Double sided foiled insulation

- WALL FINISHES

  - CHB

    - Exterior CHB Walls

      - Plain cement plaster painted finish;

      - Base Preparation: Build Rite Shelltect White Hydrophobic, or approved equal;

      - Base Coat: Boysen Acrylic Flat Latex Paint, or approved equal, 2 coats;

      - Top Coat: Boysen Acrylic Semi-Gloss Paint, or approved equal, 1 coat

    - Interior CHB Walls at Common Areas

      - Plain cement plaster painted finish;

      - Base Preparation: Build Rite Skim Coat SF, or approved equal;

      - Base Coat: Boysen Acrylic Flat Latex Paint, or approved equal, 2 coats;

      - Top Coat: Boysen Acrylic Semi-Gloss Paint, or approved equal, 1 coat

    - Interior CHB Walls at all Bedroom Units (as delivered)

      - Plain cement plaster painted finish;

      - Base Preparation: Build Rite Skim Coat SF, or approved equal;

      - Base Coat: Boysen Acrylic Flat Latex Paint, or approved equal, 2 coats;

  - 200 mm X 300 mm GLAZED CERAMIC WALL TILES

    - Location: Residential Unit Toilet Walls.

    - Type: Melrose Quadro by Floor Center or approved equal.

  - 600 mm X 1200 mm POLISHED CONCRETE WALL TILES

    - Location: Front Exterior

    - Type: Polished Concrete Tiles by Skymart or approved equal.

  - (1’ x 8’)  WPC OUTDOOR FLUTED WALL PANEL or approved equal

    - Location: Front Exterior

    - Type: Dark wood pvc wall panel or approved equal.

- FLOOR FINISHES

  - (FF-01) 600 mm X 600 mm STONE TILES

    - Location: Porch, MBR Balcony

    - Type: M1554 by Floor Center or approved equal

  - (FF-02) 600 mm X 600 mm PORCELAIN TILES

    - Location: Foyer, Multi- purpose Rm. 2

    - Type: M100-7 by Floor Center or approved equal

  - (FF-03) 300 mm X 300 mm NON- SKID CERAMIC TILES

    - Location: Service Area T&B, Laundry Area

    - Type: I-3474 by Floor Center or approved equal.

  - (FF-04) 600mm X 1200mm PORCELAIN TILES

    - Location: Living Rm., Dining Rm., Kitchen

    - Type: Y1GL17GE by Floor Center or approved equal

  - (FF-05) 200mm X 1200mm NON SKID CERAMIC TILES

    - Location: 2F Balcony, Family Rm.

    - Type: 201217-J by Floor Center or approved equal

  - (FF-06) 600mm X 600mm NON SKID CERAMIC TILE

    - Location: Bedroom T&B

    - Type: CL07 by Floor Center or approved equal

  - (FF-07) 500 mm x 500 mm HIGH DENSITY TILED MATS

    - Location: Gym

    - Type: by GOMMA Rubber Flooring Solutions or approved equal

  - (FF- 08) (150 MM X 900 MM) WOOD VINYL TILES

    - Location: All 3F Bedrooms

    - Type: LD189-03 by Floor Center or approved equal

  - (FF- 09) (200 MM X 1200 MM) NON SKID CERAMIC TILES

    - Location: 3F LIVING AREA

    - Type: 201201-J by Floor Center or approved equal

- CEILING FINISHES

  - CEILING BOARDS

    - 1/2” Gypsum Board on Metal Framing

      - Location:     All rooms except T&B;

      - Base Coat: Boysen Acrylic Flat Latex Paint, or approved equal, 2 coats;

      - Top Coat: Boysen Acrylic Flat Latex Paint, or approved equal, 1 coat

    - 1/4” Moisture Resistant Gypsum Board on Metal Framing

      - Location: All Toilet Ceiling;

      - Base Coat: Boysen Acrylic Flat Latex Paint, or approved equal, 2 coats;

      - Top Coat: Boysen Acrylic Semi- Gloss Paint, or approved equal, 1 coat

  - PVC CEILING PANEL

    - PVC Ceiling Panel on Metal Framing

      - Location: Gym, Living Rm., Dining Rm.,Family Rm.,  All Bedrooms

      - Finish: Wood design or approved equal

- DOORS

  - (D1) WOODEN PANEL DOOR

    - Size : 2,000 mm X 2,450 mm including door frame

    - Location: Main Door Foyer

    - Door Frame: 2" x 6" K/D wooden door jamb in varnish finish; Sample and color to be approved by the Architect

    - Accessories: Provide sample for door handle, lockset and hinges by Schlage or approved equal

  - (D2) WOODEN FLUSH DOOR

    - Size : 900 mm X 2,150 mm including door frame

    - Location: Multi- purpose Rm.

    - Door Frame: 2" x 4" K/D wooden door jamb in varnish finish; Sample and color to be approved by the Architect

    - Accessories: Provide sample for door handle, lockset and hinges by Schlage or approved equal

  - (D3) WOODEN FLUSH DOOR

    - Size : 750 mm X 2,150 mm including door frame

    - Location: T&B

    - Door Frame: 2" x 4" K/D wooden door jamb in varnish finish; Sample and color to be approved by the Architect

    - Accessories: Provide sample for door handle, lockset and hinges by Schlage or approved equal

  - (D4) WOODEN FLUSH DOOR

    - Size : 800 mm X 2,150 mm including door frame

    - Location: Linen Rm., Driver’s Quarter, Maid’s Quarter

    - Door Frame: 2" x 4" K/D wooden door jamb in varnish finish; Sample and color to be approved by the Architect

    - Accessories: Provide sample for door handle, lockset and hinges by Schlage or approved equal

  - (D5) WOODEN FLUSH DOOR

    - Size : 1,000 mm X 2,150 mm including door frame

    - Location: Service Area

    - Door Frame: 2" x 6" K/D wooden door jamb in varnish finish; Sample and color to be approved by the Architect

    - Accessories: Provide sample for door handle, lockset and hinges by Schlage or approved equal

  - (D6) SLIDING GLASS DOOR

    - Size: 1,700 mm X 2,450 mm including door jamb

    - Location: Family Rm.

    - Door Frame: 	Extruded Aluminum Section, Powder coated finish;  Sample and color to be approved by the Architect

    - Glass: 12 mm thk. Polished Tempered glass. Submit sample for Architect’s approval

    - Accessories: Provide sample for door handle, lockset and hinges by Schlage or approved equal

  - (D7) SLIDING GLASS DOOR

    - Size: 2,100 mm X 2,450 mm including door jamb

    - Location: 2F Balcony

    - Door Frame: 	Extruded Aluminum Section, Powder coated finish;  Sample and color to be approved by the Architect

    - Glass: 12 mm thk. Polished Tempered glass. Submit sample for Architect’s approval

    - Accessories: Provide sample for door handle, lockset and hinges by Schlage or approved equal

  - (D8) WOODEN FLUSH DOOR

    - Size : 800 mm X 2,150 mm including door frame

    - Location: WIC

    - Door Frame: 2" x 4" K/D wooden door jamb in varnish finish; Sample and color to be approved by the Architect

    - Accessories: Provide sample for door handle, lockset and hinges by Schlage or approved equal

  - (D9) WOODEN FLUSH DOOR

    - Size : 900 mm X 2,150 mm including door frame

    - Location: Bedrooms, Kitchen Stair

    - Door Frame: 2" x 4" K/D wooden door jamb in varnish finish; Sample and color to be approved by the Architect

    - Accessories: Provide sample for door handle, lockset and hinges by Schlage or approved equal

- WINDOWS

  - (W1) FIXED GLASS WINDOW

    - Size: 2,490 mm X 6,480 mm including jamb

    - Location: Main Staircase

    - Glass: 8mm thk. tempered glass

    - Window Frame: powder coated aluminum frame or approved equal.

  - (W2) COMBINED FIXED AND SLIDING WINDOW

    - Size : 1,200 mm X 900 mm including jamb

    - Location: Maid’s Quarter

    - Glass: 6mm thk. tempered glass

    - Window Frame: powder coated aluminum frame or approved equal.

  - (W3) AWNING WINDOW

    - Size : 500  mm X 900 mm including jamb

    - Location: Toilet and Bath

    - Glass: 6mm thk. tempered glass

    - Window Frame: powder coated aluminum frame or approved equal.

  - (W4) SINGLE HUNG WINDOW

    - Size : 600  mm X 1,200 mm including jamb

    - Location: Bedroom 2

    - Glass: 6mm thk. tempered glass

    - Window Frame: powder coated aluminum frame or approved equal.

  - (W5) SLIDING WITH FIXED  WINDOW

    - Size : 1,000  mm X 1,200 mm including jamb

    - Location: Laundry Area, Linen Rm.

    - Glass: 6mm thk. tempered glass

    - Window Frame: powder coated aluminum frame or approved equal.

  - (W6) FIXED WITH AWNING WINDOW

    - Size : 900 mm X 1,500 mm including jamb

    - Location: Multi- purpose Rm., Family Rm., Kitchen, 3F Living Rm., Bedroom 4

    - Glass: 6mm thk. tempered glass

    - Window Frame: powder coated aluminum frame or approved equal.

  - (W7) FIXED WITH AWNING WINDOW

    - Size : 3,000 mm X 1,500 mm including jamb

    - Location: 2F Living Area

    - Glass: 6mm thk. tempered glass

    - Window Frame: powder coated aluminum frame or approved equal.

  - (W8) AWNING WINDOW

    - Size : 500  mm X 600 mm including jamb

    - Location: Toilet and Bath

    - Glass: 6mm thk. tempered glass

    - Window Frame: powder coated aluminum frame or approved equal.

  - (W9) FIXED WITH SLIDING  WINDOW

    - Size : 1,200  mm X 1,200 mm including jamb

    - Location: Kitchen

    - Glass: 6mm thk. tempered glass

    - Window Frame: powder coated aluminum frame or approved equal.

  - (W10) FIXED WITH SLIDING  WINDOW

    - Size : 500  mm X 1,800 mm including jamb

    - Location: 2F Service Stairs

    - Glass: 6mm thk. tempered glass

    - Window Frame: powder coated aluminum frame or approved equal.

  - (W11) FIXED WITH AWNING  WINDOW

    - Size : 2,100  mm X 1,200 mm including jamb

    - Location: Dining Area, Bedroom 4

    - Glass: 6mm thk. tempered glass

    - Window Frame: powder coated aluminum frame or approved equal.

  - (W12) FIXED WITH AWNING  WINDOW

    - Size : 1,500 mm X 3,050 mm including jamb

    - Location: Family Rm.

    - Glass: 6mm thk. tempered glass

    - Window Frame: powder coated aluminum frame or approved equal.

  - (W13) FIXED WITH AWNING  WINDOW/ CORNER WINDOW

    - Size : 1,500 mm X 2,050 mm including jamb

    - Location: Bedroom 2

    - Glass: 10 mm thk. tempered glass

    - Window Frame: powder coated aluminum frame or approved equal.

  - (W14) FIXED WITH AWNING  WINDOW/ CORNER WINDOW

    - Size : 1,500 mm X 1,750 mm including jamb

    - Location: Bedroom 2

    - Glass: 10 mm thk. tempered glass

    - Window Frame: powder coated aluminum frame or approved equal.

  - (W15) FIXED WITH AWNING  WINDOW

    - Size : 1,500 mm X 1,800 mm including jamb

    - Location: Bedroom 3

    - Glass: 6 mm thk. tempered glass

    - Window Frame: powder coated aluminum frame or approved equal.

  - (W16) FIXED WITH AWNING  WINDOW

    - Size : 1,500 mm X 2,400 mm including jamb

    - Location: Master Bedroom

    - Glass: 6 mm thk. tempered glass

    - Window Frame: powder coated aluminum frame or approved equal.

  - (W17) SLIDING  WINDOW

    - Size : 1,500 mm X 2,700 mm including jamb

    - Location: Gym

    - Glass: 6 mm thk. tempered glass

    - Window Frame: powder coated aluminum frame or approved equal.

- FIXTURES

  - All Plumbing Fixture shall be HCG, or approved equal.

    - Water Closet shall be HCG C403 Osiris (2015/2016 catalogue) or approved equal

    - Lavatory shall be HCG L61 Saturn II (2015/2016 catalogue) or approved equal

  - All lavatory faucets shall be American Standard, or approved equal

  - Bathroom accessories not included

- KITCHEN CABINET

  - Type: Modular Cabinet

  - Top Surface: natural granite with 4” backsplash, Submit sample for Architect’s approval; Stainless Steel Kitchen Sink 60 x 90cm

  - Exterior Side Panels: Laminated Siding shall be used as defined in the Design Drawings or as maybe approved

  - Components / Accessories: Goose neck Faucet and Base Cabinet

  - Handle, Strainer, Grease Trap, and P-trap or approved equal